Raymond Costa

CS-350

02/19/2026

7-1 Final Project: Smart Thermostat Prototype Report

Introduction

SysTec is preparing to enter the rapidly expanding smart thermostat market, projected to reach nearly $9 billion by 2026 (Fortune Business Insights, 2023). To support this initiative, a functional prototype was developed to demonstrate the core embedded capabilities required for a production-ready thermostat. This prototype focuses on low-level hardware interaction, including temperature sensing, user input, LED indicators, LCD output, and UART-based communication. The goal is to validate the foundational embedded behavior before transitioning to a Wi-Fi-enabled cloud architecture in the next development phase.

Peripheral Support

The prototype supports all required peripherals using the Raspberry Pi 4B. The AHT20 temperature sensor communicates over the I²C bus, allowing accurate and continuous temperature readings (Adafruit, 2023). Three GPIO-connected buttons provide user input: one cycles between OFF, HEAT, and COOL states, while the other two adjust the temperature set point. Two PWM-controlled LEDs indicate heating and cooling behavior, fading when active and remaining solid when the set point is reached. A 16×2 LCD display connected through digital I/O provides real-time feedback, including date, time, current temperature, and system state. Finally, UART communication simulates cloud data transmission by sending a comma-delimited status string every 30 seconds. Together, these peripherals demonstrate the complete embedded functionality required for a smart thermostat.

Cloud Connectivity Requirements

Although the prototype uses UART to simulate cloud communication, the production thermostat must support Wi-Fi. This requires an architecture capable of secure wireless networking, periodic data transmission, and integration with SysTec’s analytics platform. The thermostat must reliably send temperature readings, set points, and system state to the cloud. It must also support over-the-air updates, authentication, and encrypted communication (Microchip Technology, 2022). The UART-based simulator validates the data format and transmission logic, forming the basis for the Wi-Fi implementation in the next phase.

Architecture Comparison

Three hardware architectures were evaluated for production: Raspberry Pi, Microchip microcontrollers, and Freescale (NXP) microcontrollers.

Raspberry Pi: The Raspberry Pi 4B offers extensive RAM, Flash storage, and built-in Wi-Fi. It supports all required peripherals and provides a full Linux environment (Raspberry Pi Foundation, 2023). However, it is more expensive, consumes more power, and is over-provisioned for a thermostat. Its general-purpose nature makes it ideal for prototyping but less optimal for mass production.

Microchip (PIC/ATSAMD/ESP32-based): Microchip microcontrollers offer low-power operation, integrated Wi-Fi options such as the WINC1500 module, and sufficient Flash/RAM for embedded thermostat logic (Microchip Technology, 2022). They support I²C, UART, PWM, and GPIO, making them well-suited for the required peripherals. Their cost efficiency and reliability make them strong candidates for production.

Freescale/NXP (Kinetis, i.MX RT): NXP microcontrollers provide robust peripheral support, strong real-time performance, and optional integrated wireless modules (NXP Semiconductors, 2023). They offer more processing power than typical Microchip MCUs while remaining cost-effective. Their Flash and RAM capacities are more than adequate for thermostat firmware.

Recommendation

For production, a Microchip-based architecture is recommended. It meets all business requirements: support for I²C, UART, PWM, and GPIO; integrated Wi-Fi capability; and sufficient Flash and RAM for the thermostat firmware. It also offers low cost, low power consumption, and strong reliability, all key factors for a consumer thermostat. While Raspberry Pi is ideal for prototyping, Microchip provides the best balance of performance, cost, and scalability for SysTec’s long-term product strategy.

Conclusion

The prototype successfully demonstrates the embedded functionality required for a smart thermostat. It integrates temperature sensing, user input, LED indicators, LCD output, and UART communication into a cohesive system driven by a state machine. The next phase will transition from UART simulation to Wi-Fi-enabled cloud connectivity. Based on the analysis of hardware architectures, Microchip microcontrollers offer the most suitable platform for production deployment, aligning with SysTec’s technical and business objectives.

References

Adafruit. (2023). AHT20 temperature and humidity sensor guide. https://learn.adafruit.com

Fortune Business Insights. (2023). Smart thermostat market size, share & trends. https://www.fortunebusinessinsights.com

Microchip Technology. (2022). WINC1500 low-power Wi-Fi module product brief. https://www.microchip.com

NXP Semiconductors. (2023). Kinetis and i.MX RT crossover MCUs overview. https://www.nxp.com

Raspberry Pi Foundation. (2023). Raspberry Pi 4 Model B product documentation. https://www.raspberrypi.com